# Architectural Plan Verification Checklist

Residential Building (Basement, Ground Floor, First Floor) | Kribi, Cameroon

# 1. Document Control & Quality Process (ISO 9001 / ISO 19650)

[   ]  Version Control: Is the current drawing revision clearly numbered and dated in the title block?

[   ]  File Naming: Do the digital file names follow a standardized convention (e.g., ProjectName_Floor_Discipline_Version)?

[   ]  Approvals: Are the plans signed/stamped by the lead architect, structural engineer, and MEP engineers?

[   ]  Project Metadata: Does the title block include the exact site location, client name, scale, and paper size?

# 2. Site Plan & Environmental (Kribi Specifics)

[   ]  Boundaries & Setbacks: Do the building footprints respect the legal setbacks from the property lines as per local municipal regulations?

[   ]  Orientation & North Arrow: Is the True North arrow visible? Are living spaces oriented to minimize direct afternoon sun?

[   ]  Site Drainage: Are site slopes directed away from the building foundation to prevent water pooling?

[   ]  Site Access: Are driveways, pedestrian gates, and waste collection zones clearly marked and dimensioned?

# 3. Basement Verification (High Risk Area)

[   ]  Waterproofing/Tanking: Is full exterior waterproofing (tanking) specified for the retaining walls and basement slab?

[   ]  Structural Retaining: Are the basement walls specified as reinforced concrete to hold back earth pressure?

[   ]  Egress / Life Safety: Is there at least one emergency exit window or direct egress stairwell leading outside in case of fire?

[   ]  Ventilation: Is there a provision for mechanical ventilation or light wells/areaways for natural air circulation?

[   ]  Plumbing/Sump Pump: If basement drains are below the sewer line, is a sump pump pit designed into the floor plan?

# 4. Spatial & Structural Coordination (Ground & First Floor)

[   ]  Grid Alignment: Do the structural column grids align perfectly across the basement, ground floor, and first floor?

[   ]  Staircase Dimensions: Are the staircases identical in location across all floors? Verify tread depth (min. 280mm) and riser height (max. 180mm).

[   ]  Headroom Clearance: Is there a minimum of 2.1 meters (7 feet) of vertical clearance at every point on the staircase?

[   ]  Wall Alignment: Do the heavy load-bearing masonry walls on the first floor sit directly above load-bearing walls or beams?

[   ]  Shaft Alignment: Do the plumbing shafts stack directly over the ground floor/basement to allow straight pipe drops?

# 5. Life Safety & Accessibility (IBC / Local Code)

[   ]  Egress Paths: Is the distance from the furthest bedroom to the main staircase under 15-20 meters?

[   ]  Balcony Safety: Are first-floor balcony parapets/handrails detailed at a minimum height of 1.05 meters?

[   ]  Door Swings: Do doors open in logical directions without blocking corridors?

[   ]  Fire Separation: Is the basement separated from the ground floor by a fire-rated door?

# 6. Drafting & Presentation Consistency (ISO 128)

[   ]  Line Weights: Are cut walls heavily weighted, while furniture and background elements are light?

[   ]  Dimension Chains: Are there three clear tiers of dimensions? (Overall length, Grid-to-grid, Window/door openings)

[   ]  Room Tags: Does every room have a tag indicating Room Name, Area, and Floor Finish?

[   ]  Section Markers: Do the section cut lines correspond accurately to the generated section drawings?