# Energy Supply Verification Documentation Checklist

Residential Building (Basement, G+1) | Kribi, Cameroon
Standards Reference: IEC 60364, IEC 60947, ISO 50001, ENEO Norms

# 1. Main Grid Supply & Metering (IEC 60364-6 & ENEO Norms)

[   ]  Single Line Diagrams (SLD): Ensure final 'As-Built' electrical schematics accurately reflect the installed main distribution boards and circuit breaker ratings.

[   ]  Commissioning Service Record: Obtain signed documentation for the utility energy meter installation, logging the meter identification, CT ratios, and operational checks.

[   ]  Insulation & Earthing Certificates: Mandate formal test reports proving that ground resistance meets local standards (crucial in Kribi's wet coastal soil) and that cable insulation is intact.

# 2. Backup Power & Transfer Switching (IEC 60947-6-1)

[   ]  Generator Acceptance Tests: Require both Factory Acceptance Test (FAT) and Site Acceptance Test (SAT) reports for the backup generator to verify load capacity and exhaust ventilation.

[   ]  ATS Functionality Log: Ensure there is documented proof that the Automatic Transfer Switch (ATS) seamlessly shifts between grid and generator power without phase conflicts or back-feeding.

[   ]  Fuel Storage Safety Approval: Verify that the diesel storage tank complies with local fire safety and environmental containment regulations.

# 3. Solar PV Integration (IEC 60364-7-712)

[   ]  Inverter Start-Up Report: Require a formal log of the inverter's commissioning, including DC/AC voltage readings, anti-islanding test results, and surge protection verification.

[   ]  Array Wiring Verification: Document that all DC disconnects, combiner boxes, and PV wiring are rated for high-temperature and UV exposure, and torque-checked according to manufacturer specifications.

[   ]  Grid-Tie Compliance: If feeding power back into the grid, ensure the documentation includes the necessary utility approvals and smart-meter configuration logs.

# 4. System Handover & Energy Management (ISO 50001)

[   ]  O&M Manuals: Ensure all Operations and Maintenance manuals for main switches, generators, and solar equipment are compiled in a single, accessible handover binder.

[   ]  Load Balancing Report: Document that the electrical load is evenly distributed across the three phases to prevent uneven heating or breaker tripping.

[   ]  User Training Sign-Off: Require a signed form proving the facility manager or homeowner has been formally trained on emergency shut-offs and system resets.