# Foundation Construction Verification Checklist

Residential Building (Basement, G+1) | Kribi, Cameroon
Standards Reference: Eurocode 2 (EN 1992), Eurocode 7 (EN 1997), ISO 9001 Construction Quality

# Stage 1: Post-Reinforcement (Before Pouring Ground / Tie Beams)

[   ]  Grid & Alignment: Are the ground beam formworks perfectly aligned with the surveyor's established grid lines and property boundaries?

[   ]  Rebar & Lapping: Are longitudinal reinforcement bars and shear links (stirrups) sized, spaced, and lapped exactly as per the structural drawings?

[   ]  Concrete Cover (Crucial for Kribi): Are concrete or PVC spacer blocks installed to guarantee adequate clear cover (typically 40-50mm for foundations) protecting the steel from damp soil?

[   ]  MEP Sleeves & Inserts: Are PVC sleeves securely installed through the beam rebar for plumbing/electrical lines to prevent future hacking/breaking of the concrete?

[   ]  Formwork Integrity: Is the formwork oiled, rigidly braced, and leak-proof to prevent cement grout from escaping during vibration?

# Stage 2: Pre-Ground Floor Casting (Sub-base, DPM & Mesh)

[   ]  Backfill Compaction: Has the laterite or selected earth backfill been mechanically compacted in layers and passed the necessary compaction (density) tests?

[   ]  Anti-Termite Treatment: Has the compacted soil been treated with approved anti-termite chemicals before laying the blinding concrete?

[   ]  Damp Proof Membrane (DPM): Is a heavy-duty polyethylene sheet (DPM) laid over the blinding concrete with taped overlaps (min. 150mm) to block rising soil moisture?

[   ]  Under-Slab MEP: Are all under-slab plumbing drainage pipes and electrical conduits securely fixed, sloped correctly, and pressure-tested before being covered?

[   ]  Slab Reinforcement: Is the steel wire mesh (or rebar) placed on spacers so it sits at the correct height within the slab thickness, rather than lying flat on the DPM?

# Stage 3: Post-Ground Floor Casting (Curing & Column Prep)

[   ]  Levelness & Finish: Was the wet concrete properly vibrated and screeded to the exact Finished Floor Elevation (FFE) required by the architectural plans?

[   ]  Column Starter Bars (Dowels): Are the dowel bars for the ground-floor columns perfectly vertical, accurately positioned on the grid, and free of dried concrete splatter?

[   ]  Curing Regime: Is a strict curing process (ponding, wet hessian cloth, or curing compound) initiated and maintained for at least 7 days to prevent thermal cracking in the tropical heat?

[   ]  Visual Defect Inspection: Is the fully cured slab inspected for and free of severe plastic shrinkage cracks, honeycombing at the edges, or uneven surfaces holding water?

[   ]  Construction Joints: If the slab was poured in sections, are the construction joints located at structurally safe points and properly prepared with waterstops if required?