# MEP Plan Verification Checklist

Residential Building (Basement, G+1) | Kribi, Cameroon
Standards Reference: NF C 15-100, NF DTU 60, ASHRAE, IEC/IPC

# 1. Electrical Plan Verification (Ref: NF C 15-100 / IEC 60364)

[   ]  Panel Locations: Ensure the main distribution boards are easily accessible, adequately sized, and strictly located away from wet areas (bathrooms, basement sumps).

[   ]  Earthing/Grounding: Verify that a comprehensive grounding system (earth pit details and rod specifications) is detailed, which is critical in Kribi's humid, highly conductive coastal environment.

[   ]  Load Schedules: Check that the single-line diagram includes accurate load calculations, breaker sizing, and appropriate cable cross-sections for residential appliances.

[   ]  Conduit Routing: Confirm that electrical conduit routing within the concrete slabs does not cluster too densely, which could compromise the structural integrity of the slabs or beams.

# 2. Plumbing Plan Verification (Ref: NF DTU 60.1 / 60.11 / IPC)

[   ]  Slopes and Inverts: Verify that all horizontal drainage pipes indicate a minimum slope (typically 1.5% to 2%) to ensure gravity flow to the septic system or municipal sewer.

[   ]  Basement Drainage: Ensure the basement includes a specified sump pump pit, pump capacity, and backwater valves to prevent flooding from Kribi's high water table.

[   ]  Ventilation Stacks: Confirm that all plumbing waste lines are properly vented through the roof to prevent trap siphoning and sewer gas buildup inside the house.

[   ]  Water Supply Routing: Check that cold and hot water supply lines are routed efficiently, with clearly marked isolation valves for each bathroom and kitchen zone.

# 3. Mechanical (HVAC) Plan Verification (Ref: ASHRAE / NF EN)

[   ]  Equipment Placement: Verify that outdoor AC compressor units are positioned for optimal airflow and specify anti-corrosion coatings to withstand Kribi's salty coastal air.

[   ]  Basement Ventilation: Ensure adequate mechanical exhaust and fresh air intake are designed for the basement to prevent mold, condensation, and stale air.

[   ]  Service Shafts: Check that AC refrigerant lines and condensate drains have dedicated architectural shafts and do not clash with structural beams.

[   ]  Exhaust Systems: Confirm that all enclosed bathrooms and the kitchen have dedicated mechanical extractors that vent directly to the building's exterior.