# Soil Test & Geotechnical Verification Report

Project: Residential Building (Basement, G+1)
Location: Kribi, South Region, Cameroon
Mandating Codes: Eurocode 7 (EN 1997-1/2), IBC Chapter 18 (Soils & Foundations)

--------------------------------------------------------------------------------

# 1. Regulatory Mandates & Context

Geotechnical investigations are legally and technically mandated for any structure incorporating a basement, particularly in coastal zones with high water tables like Kribi. Eurocode 7 (EN 1997-2) mandates specific field and laboratory testing to ensure ground stability. The International Building Code (IBC Chapter 18) strictly dictates that buildings with basements must have soil tested for hydrostatic pressure, lateral earth pressure, and chemical aggressiveness to protect the concrete foundation.

## 2. Sub-Stage 1: Field Investigation Verification

| Verification Element | Required Standard / Code Mandate | Methodology | Status |
| --- | --- | --- | --- |
| Borehole Depth & Spacing | Minimum of 3 boreholes for a footprint < 500m². Depth must extend beneath the basement level (approx. 10m-15m) into stable bearing strata. (Eurocode 7, Section 2) | Review driller logs and GPS coordinates of boreholes to ensure they cover the building footprint. | [ ] Pass [ ] Fail |
| Standard Penetration Test (SPT) | SPT N-values must be recorded at 1.5m intervals to determine soil density and shear strength. (EN ISO 22476-3) | Review field logs for continuous N-value data down to the termination depth. | [ ] Pass [ ] Fail |
| Groundwater Table Logging | Exact static water level must be recorded 24 hours after drilling to determine hydrostatic pressure against the basement. (IBC 1803.5.4) | Verify the water table depth (often shallow in Kribi) is explicitly stated in the field report. | [ ] Pass [ ] Fail |

## 3. Sub-Stage 2: Laboratory Testing Verification

| Verification Element | Required Standard / Code Mandate | Methodology | Status |
| --- | --- | --- | --- |
| Chemical Aggressiveness | Soil and groundwater must be tested for sulfates, chlorides, and pH to dictate cement type and rebar cover. (Eurocode 2 / EN 206-1) | Review laboratory chemical analysis report. High chlorides require marine-grade concrete. | [ ] Pass [ ] Fail |
| Atterberg Limits | Plasticity Index (PI) and Liquid Limit (LL) must be defined to check for expansive clays (which swell and crack basements). (IBC 1803.5.3) | Verify laboratory soil classification (e.g., USCS classification). | [ ] Pass [ ] Fail |
| Consolidation & Shear Strength | Oedometer and Triaxial/Direct Shear tests to predict how much the building will sink (settlement) over 50 years. (EN 1997-2) | Review settlement calculations and cohesion (c) / friction angle (phi) parameters. | [ ] Pass [ ] Fail |

## 4. Sub-Stage 3: Final Geotechnical Report Parameters

| Verification Element | Required Standard / Code Mandate | Methodology | Status |
| --- | --- | --- | --- |
| Allowable Bearing Capacity | Report must state the Safe Bearing Capacity (SBC) in kPa/MPa to allow structural engineers to size the footings. (Eurocode 7, Section 6) | Cross-reference the recommended SBC with the structural engineer's foundation drawings. | [ ] Pass [ ] Fail |
| Lateral Earth Pressure Parameters | Report must provide active, passive, and at-rest earth pressure coefficients (Ka, Kp, Ko) to design the basement retaining walls. (IBC 1803.5.5) | Verify these specific coefficients are present for the structural retaining wall design. | [ ] Pass [ ] Fail |
| Foundation Type Recommendation | Clear directive on whether isolated, raft, or pile foundations are required based on the loads and soil strength. | Verify structural plans align with the geotechnical engineer’s final recommendation. | [ ] Pass [ ] Fail |

# 5. Structural/Geotechnical Engineer Sign-Off

By signing below, I certify that the geotechnical investigation report complies with Eurocode 7 and IBC Chapter 18 standards. The field and laboratory data is sufficient to safely design the foundation and basement retaining structures against local soil bearing, hydrostatic, and chemical conditions.

Name of Verifying Engineer: _______________________________________

Professional License Number: ______________________________________

Date of Verification: ______________________________________________

Signature & Stamp: ___________________________