Structural Phase Verification: Specifics and Methods

Project: Residential Building (Basement, G+1)
Location: Kribi, South Region, Cameroon
Scope: Superstructure (Columns, Beams, Suspended Slabs, Staircases)
Standards Reference: Eurocode 2 (EN 1992), ISO 1803 (Building Tolerances)

--------------------------------------------------------------------------------

This report outlines the strict verification checkpoints, allowable tolerances, and exact field methodologies required to approve the superstructure construction phases. No structural element may be poured or loaded until the respective phase is signed off.

1. Formwork & Falsework Verification (Pre-Pour)

| Structural Element / Check | Specification / Tolerance | Verification Method & Tool | Status/Sign-off |
| --- | --- | --- | --- |
| Column/Wall Plumbness (Verticality) | Maximum deviation: 5mm per 3m height. | Heavy plumb bob dropped from top edge OR Laser Level projection. | [ ] Pass [ ] Fail |
| Beam Soffit Level & Camber | Matches Finished Floor Elevation (FFE). Camber built-in if span > 5m. | Auto-Level or Total Station reading taken at span ends and mid-span. | [ ] Pass [ ] Fail |
| Slab Shoring & Propping | Props spaced at max 900mm. Strictly plumb. Resting on solid bases. | Visual inspection, tape measure for spacing, and spirit level for prop plumbness. | [ ] Pass [ ] Fail |
| Formwork Tightness & Release | Joints sealed against grout loss. Mold oil applied. | Visual inspection. Flashlight check through joints from beneath. | [ ] Pass [ ] Fail |

2. Steel Reinforcement Verification (Pre-Pour)

| Structural Element / Check | Specification / Tolerance | Verification Method & Tool | Status/Sign-off |
| --- | --- | --- | --- |
| Bar Diameter & Grade | Matches structural plans (e.g., T12, T16). Grade 400/500 MPa. | Digital calipers to verify diameter; visual check of mill marks on steel. | [ ] Pass [ ] Fail |
| Lap Splice Lengths | Min. 40x to 50x bar diameter, located in compression zones. | Steel tape measure directly along the lapped sections. | [ ] Pass [ ] Fail |
| Stirrup (Link) Spacing | As specified (e.g., closer spacing near column/beam joints for shear). | Tape measure across a 1-meter span to count total links. | [ ] Pass [ ] Fail |
| Concrete Cover | 25mm - 30mm for superstructure (internal). 40mm for external/wet areas. | Physical count and measurement of PVC/concrete spacer blocks tied to the rebar grid. | [ ] Pass [ ] Fail |

3. Post-Stripping Quality & Defect Verification (Post-Pour)

| Structural Element / Check | Specification / Tolerance | Verification Method & Tool | Status/Sign-off |
| --- | --- | --- | --- |
| Honeycombing & Voids | No exposed rebar. No voids deeper than cover depth. | Visual inspection. Light tapping with a sounding hammer to detect internal delamination. | [ ] Pass [ ] Fail |
| Dimensional Accuracy | Cross-sections within ±5mm of plan dimensions. | Digital laser measure or steel tape across column/beam faces. | [ ] Pass [ ] Fail |
| Plastic Shrinkage Cracks | Crack widths must not exceed 0.3mm (durability limit). | Direct measurement using a standardized transparent Crack Width Ruler/Gauge. | [ ] Pass [ ] Fail |
| Slab Deflection (Sag) | Post-propping sag must not exceed Span/250. | String line stretched taut beneath the slab or laser level measurement. | [ ] Pass [ ] Fail |

4. Engineer Sign-Off

Name of Verifying Engineer: _______________________________________

Professional License Number: ______________________________________

Date of Verification: ______________________________________________

Signature: ___________________________