# Structural Plan Verification Checklist

Residential Building (Basement, G+1) | Kribi, Cameroon
Standards Reference: Eurocodes (EN 1991, EN 1992, EN 1997) & BAEL 91

# 1. General Notes & Specifications (Ref: Eurocode 1 & Eurocode 2)

[   ]  Material Grades: Are the concrete compressive strength and steel reinforcement grades explicitly stated? (EC2)

[   ]  Design Loads: Are the assumed dead, live, and environmental loads (wind, hydrostatic) listed? (EC1)

[   ]  Geotechnical Data: Do the general notes reference the specific soil investigation report for the Kribi site?

[   ]  Concrete Cover: Is the clear cover increased for coastal humidity and aggressive soil conditions? (EC2 Durability Requirements)

# 2. Foundation & Basement Retaining Walls (Ref: Eurocode 7 & Eurocode 2)

[   ]  Footing Dimensions: Do the isolated or raft footing sizes align with the allowable soil bearing capacity? (EC7 Geotechnical Design)

[   ]  Hydrostatic Pressure: Are the basement retaining walls designed and detailed to withstand water pressure from the high water table? (EC1 / EC7)

[   ]  Waterproofing Integration: Are waterstops specified at cold joints between the basement slab and retaining walls?

[   ]  Column Dowels: Are the dowel lengths extending from the foundation into the columns sufficient for splicing? (EC2 Detailing)

# 3. Columns & Beams - Superstructure (Ref: Eurocode 2)

[   ]  Grid Alignment: Do structural grids match the architectural plans perfectly to avoid eccentric loading?

[   ]  Load Path Continuity: Do all columns stack directly from the roof down to the foundation without unsupported offsets?

[   ]  Reinforcement Detailing: Are lap splices located away from high-stress zones (e.g., mid-height for columns)? (EC2 Detailing)

[   ]  Beam Depths: Do beam drop depths conflict with architectural ceiling heights or MEP duct routing?

# 4. Slabs & Openings (Ref: Eurocode 2)

[   ]  Slab Thickness: Is the slab thickness adequate for the spans to prevent excessive deflection? (EC2 Serviceability Limit State)

[   ]  Opening Reinforcement: Is additional diagonal and edge reinforcement detailed around stairwells and plumbing shafts? (EC2)

[   ]  Cantilevers: Is top-layer tension reinforcement correctly detailed for cantilevered first-floor balconies? (EC2)

[   ]  Step Downs: Are sunken slabs (for bathrooms/wet areas) clearly indicated with correct elevation tags?